- Definition & Purpose for no-augmentation

  - A water control structure has been releasing clearwater return flows from a hydroelectric plant into the Platte River near Lexington Nebraska for 82 years. During that time, the return channel has incised, reducing longitudinal slope and prompting a transition from braided to wandering planforms. In an effort to prevent progression of these changes downstream, reducing the suitability of Whooping Crane habitat, the Platte River Recovery Implementation Program (PRRIP) began annual full-scale sediment augmentation projects approximately one mile downstream of the clearwater return in 2017. The projects were designed to offset the sediment imbalance and slow bed erosion. After 6 years of continuous annual augmentation, PRRIP has put the project on hold for the next 5 years to monitor the channel’s response to a no-augmentation condition. The purpose of this pause is to gather data to help us separate the benefits of sediment augmentation from natural channel adjustment. Data collected during this period will inform future negotiations regarding the necessity, quantity, and potential location of sediment augmentation.

- Analysis of augmentation 2017-2022 (Evaluation of Sediment Augmentation Report, Addendum)

  - Data collected

    - Annual Topo-bathymetric LiDAR

    - Annual PRRIP aerial imagery since 2007

    - Stream gage records

    - Historic transects (USBR, Tetra Tech)

    - 2009 and 2012 PRRIP 1D HEC-RAS models

    - 2016 – 2022 2D hydraulic models

    - Historic aerial imagery

  - Analyses performed

    - Analysis of incision trends vs. predictions (Murphy et al. 2004)

    - Specific gage analysis

    - Sinuosity

    - Relative Elevation Model

    - Longitudinal Profile – Changepoint analysis

    - Wetted Width

    - Volume change analysis

  - Key findings

    - Incision has been progressing more slowly than predicted due to natural channel adjustment (reduction in bed incision and increase in sinuosity).

    - Average bed erosion reduced by 20-40k cy/yr (45%-60%) during the augmentation period in the J2 Return Channel.

    - The effects of augmentation were not detectable downstream of Overton Bridge. The reach from Overton to the Kearney Canal Diversion (KCD) continues to be dynamic without a discernable trend of aggradation or degradation.

    - Planform change continued to progress through the lower half of the J2 Channel. This was noted in continued channel consolidation and incision near station 70k.

- Analysis of no-augmentation

  - Data to collect.

    - Continue collecting

      - Annual Topo-bathymetric LiDAR

      - Annual PRRIP aerial imagery

      - Stream gage records

      - Annual 2D hydraulic model

    - New data to collect

      - Stream gage at Cook for J2 channel – USGS

        - Previous estimates of J2 Channel flow were based solely on hydro releases and did not include baseflow or breakthrough channel flow. This made these estimates highly subject to underestimation. The new gage will correct this by collecting real time flows in the lower end of the channel, helping us better understand the relationship between flow and sediment flux (See Figure 1.)

      - Stage logger in breakthrough channel

        - Recording the stage in the breakthrough channel we will allow us to further monitor flows reaching the J2 channel and estimate the amount coming from the breakthrough channel.

      - Tri-annual cross section surveys at anchor points 33-36 and Station 70k

        - Cross sectional surveys will provide sub-annual (March, July, November) elevation data throughout the reach at anchor points with historic data, and at actively transitioning locations (Station 70k). These surveys will also be used to check the accuracy of LiDAR data collected each November. See Figure 2 for cross section locations.

      - Sediment samples

        - Three bed sediment samples will be collected from each anchor point every year in November. Sieve analysis will be conducted to determine sediment size gradations. The data will provide insight into channel bed material size that will assist in development of any future sediment models.

- Study Questions

  - How much will bed erosion and incision change compared to the average of 2017-2022?

    - Volume change (Section 4.3.2)

      - Volume change estimates will be developed via LiDAR raster differencing & hydraulic modeling. Water surface elevation rasters will be used to identify areas of lateral erosion on a year-to-year basis. This will allow quantification of total, lateral, and bed erosion volumes.

      - Volume flux will be calculated by dividing the mass of volume change by time and wetted area. Similar to the distinction of discharge and specific discharge, normalizing our volume change by wetted area in each sub-reach will give us the specific transport in each sub-reach. This may further highlight sub-reaches that are in disequilibrium.

    - Long-pro elevations and change point analysis (Section 3.4 and 3.5)

      - Longitudinal profiles of the deepest point of the channel (thalweg) provide a more specific numerical measure of spatial and temporal patterns of aggradation or degradation. Performing an annual change-point analysis will provide insight into whether or not the reach is shifting towards a degradational trend that it exhibited pre-augmentation.

  - How will planform change progress without augmentation?

    - Wetted width analysis

      - Wetted width will be measured at 500-ft intervals from J2 to the Kearney Canal Diversion based on yearly hydraulic modeling results. Widths during augmentation will be compared to widths during the no-augmentation period. Prior to 2017 wetted widths can only be measured at less frequent intervals which did not indicate any temporal trend in our analysis (Section 3.6). By monitoring width at a finer scale, we might see widths remain stable or reduce in the absence of augmentation if the channel consolidates from braided to wandering planforms.

    - Sinuosity

      - We will calculate sinuosity over the full reach and over smaller sub-reaches to monitor planform change. To do this we will delineate a “low flow” thalweg along the deepest part of the channel based on LiDAR data and a “high flow” thalweg based on the modeled highest velocity path at the peak of the normal hydro-cycle (1,650 cfs). This will allow us to evaluate the effect of flow on sinuosity, as well as monitor planform change through time.

    - Slope

      - We will continue to calculate longitudinal slope along the reach and evaluate it as an indicator for planform change. Slope decrease indicates transition towards wandering planform.

  - How does sediment volume change vary with flow magnitude?

    - Normalization of volume change based on flow

      - Using newly available gage data at Cook and on the breakthrough channel we will use the method discussed in our report to normalize volume change by the flow that occurred over the time when that change occurred. This will help to identify erosion occurring due to instability (sediment imbalance) from erosion due to high flow. Understanding this relationship will also enable us to estimate the sediment deficit based on different possible flow regimes.

  - Can we calculate the sediment deficit on an annual basis using our volume change analyses?

    - Sediment budget calculations

      - Using results from the volume change analysis, we will estimate the sediment deficit each year and may be able to recommend an augmentation volume calculated to achieve sediment balance based on an assumed range of future flows. Testing this calculation with additional years of modeling will inform how confident we should be in our predictions.

- Action triggers

  - Relative elevation model (REM) analysis of area of interest (AOI-1) with depth below geomorphic grade line (DBG) ≥9ft. (Figure 3.)

    - AOI-1 will reach from Station 70k downstream to the Overton bridge. Station 70K is a transitional area where the river shifts from a meandering planform to braided (see section 3.8 of report). Within the AOI, we will compare the difference of the rasterized geomorphic grade line (GGL) (see pg. 28-29 in report attached above) and the most recent LiDAR to generate an REM. This area is becoming progressively deeper compared to the GGL and this change is currently best observed in the progression of the area ≥9 feet below the GGL. Between 2016 and 2022 this class increased from 4.62 to 15.94 acres, with most of the increase occurring along a continuous, incisional thalweg. Similar increases in area occurred at other depth classes, however, deeper depth classes are not found in AOI-1 and increases in shallower classes are the result of erosion expanding these depth classes in bank areas rather than incision progressing along the thalweg (Figure 4). By measuring the area where DBG is ≥9 ft annually within the AOI-1, we can develop a rate at which incision progresses Acresyear along the thalweg to use as an action trigger. During augmentation (2016 – 2022) the average incision progression rate was ~2 acres/year , with a maximum of ~5 acres/year since 2016 (Figure 5). Figure 6 shows change from 2016 to 2022 as an example. The progression rate is expected to increase in the absence of augmentation.

    - Action trigger structure

  - REM analysis of AOI-2 with DBG ≥5ft.

    - AOI-2 will span the first 3 miles downstream from Overton Bridge. (Figure 7.) The trigger will function in the same way as described in section a. Downstream of the Overton Bridge, the ≥5ft depth class is most demonstrative of downstream thalweg incision. The class has not shown signs of progression since 2016, however it has been shown to be highly dynamic. As an example, the class increased by nearly 12 acres in 2019 following high flows, then decreased nearly 12 acres the next year (Figure 8). Figure 9 shows the change in AOI-2 from 2016 to 2022.

    - Action trigger structure

  - Overton Bridge Gage Analysis

    - Monitoring changes at the Overton Bridge gage will allow us to leverage decades of historical measurements to evaluate changes in channel depth with flow over time at the Overton Bridge (between the J2-Channel and the Cottonwood Ranch Complex). In the 2012 Sediment Augmentation Pilot Study, changes in water surface elevations at the gage for a given flow were used as an action trigger. A different method may be necessary in this case as the changes we expect to see on a year-to-year basis of no-augmentation may not be distinguishable from historical scatter. We are currently investigating the use of shift data as an alternative.

Figure 1. Proposed Gage Locations

Figure 2. Proposed Survey Cross Sections

Figure 3. REM Analysis Area of Interest-1 (AOI-1)

Figure 4. REM Analysis Area of Interest-1 (AOI-1)

Figure 5. Incision progression rate for DBG class ≥9ft within AOI-1

2022

2016

Figure 6. Comparison of 2016 and 2022 areas with DBG≥ 9ft within AOI-1

Figure 7. REM Analysis Area of Interest 2 (AOI-2)

Figure 8. Incision progression rate for DBG class ≥5ft within AOI-2

2022

2016

Figure 9. Comparison of 2016 and 2022 areas with DBG≥ 5ft within AOI-2